get/set VGAreg, a simple VGA register hacking program

=====================================================

This little tool is really included here for those who cannot resist to hack

around with their VGA cards, without really having to know how to access

them from Linux.

It should be considered even MORE dangerous than SVGATextMode ;-) If that is

possible...

It is FAR from complete, and should get a complete re-write. But it helped

me to test some stuff I needed for SVGATextMode. Anyone tempted to do better

is hereby invited to do so.

The syntax is quite easy:

setVGAreg: version 1.5. (c) 1995,1996,1997 Koen Gadeyne.

Usage: ../setVGAreg [options] VGA_register_set [register_index] data

Options: -h  print usage information

-n  Don't program VGA hardware

-d  print debugging information

-u  unlock chipset-specific registers

(needs SVGAtextMode config file)

You will need this to avoid a core dump on some cards.

-p  produce 'pipeable' decimal output (= just numbers, no text)

-x  produce 'pipeable' hex output

-t <ConfigFile>

Use <ConfigFile> instead of the default (/etc/TextConfig)

(only useful for '-u' option)

-s  `data' is a bitmask for setting bits in the register

-c  `data' is a bitmask for clearing bits in the register

register_index: An index in the specified VGA_register_set,

In decimal (e.g. '24'), hex ('0x18') or octal ('030').

Only needed when it is an indexed (indirect) VGA register.

data: the data to program into the specified register (dec|hex|oct).

(or the bitmask for the `-s' (set) or `-c' (clear) options)

VGA_register_set: any of the following:

`CRTC' `SEQ' `ATRCTL' `GRCTL' `MISC' `DAC_STATUS' `DAC_MASK' `DAC' `ET6000' `idx' `io'

getVGAreg: version 1.5. (c) 1995,1996,1997 Koen Gadeyne.

Usage: ../getVGAreg [options] VGA_register_set [register_index]

Options: -h  print usage information

-n  Don't program VGA hardware

-d  print debugging information

-u  unlock chipset-specific registers

(needs SVGAtextMode config file)

You will need this to avoid a core dump on some cards.

-p  produce 'pipeable' decimal output (= just numbers, no text)

-x  produce 'pipeable' hex output

-t <ConfigFile>

Use <ConfigFile> instead of the default (/etc/TextConfig)

(only useful for '-u' option)

register_index: An index in the specified VGA_register_set,

In decimal (e.g. '24'), hex ('0x18') or octal ('030').

Only needed when it is an indexed (indirect) VGA register.

VGA_register_set: any of the following:

`CRTC' `SEQ' `ATRCTL' `GRCTL' `MISC' `DAC_STATUS' `DAC_MASK' `DAC' `ET6000' `idx' `io'

The VGA_register_set is a mnemonic reference to an actual register in the

VGA chip. Most people are used to referring to hex IO addresses instead of

these names. Below is a table that cross-references between both notations.

Most VGA registers use an 'index-data' addressing system, where e.g. 0x3C4

is the base address of the sequencer registers. In that address, the index

into the array of sequencer registers is programmed, and in the address just

above it (0x3C5 in this example) the actual data is stored.

In the table below, if there is a checkmark in the "indexed addressing"

column, this means this VGA register uses the indexed method, and thus the

<base_address> (next column) is the address to the index register, while

<base_address+1> is the data register.

For indexed addressing registers, getVGAreg and setVGAreg will require an

index address in addition to the mnemonic address designation.

As you can see, VGA is a fine mess.

mnemonic	indexed addressing		base_address

`CRTC'		        *		0x3D4 (color); 0x3B4 (mono)

`SEQ'			*			0x3C4

`ATRCTL'		*			0x3C0

0x3C1 (read data) 0x3C0 (write data)

`GRCTL'			*			0x3CE

`MISC'					0x3CC (read); 0x3C2 (write)

`DAC_STATUS'					0x3C7 (read only)

`DAC_MASK'					0x3C6

`DAC'					0x3C7 (index for reading)

0x3C8 (index for writing)

0x3C9 (data register)

`ET6000'		* 		ET6000 PCI configuration space

`idx'			*		as specified on command line.

data register is assumed to be on

I/O address "index+1",

as in most VGA registers

`io'					direct I/O address (not indirect)

An example get/setVGAreg session could be:  ("> " denotes user input, the

rest is output from the program)

> getVGAreg CRTC 0x09

VGA 'CRTC' register, index 9 (=0x9) contains 107 (=0x6b)

> setVGAreg CRTC 0x09 0xeb

VGA 'CRTC' register, index 9 (=0x9) contains 235 (=0xeb)

Which will do something nice to your screen... ;-) (no harm done, don't be

scared).

The '-p' and '-h' options output just one number (the contents of the

register). This is useful when used in a pipe, or in a script (this one was

for an ET4000 special register, hence the '-u' flag for unlocking extended

registers):

#!/bin/bash

ORIG31=`/sbin/getVGAreg -up CRTC 0x31`

echo Original CRTC reg 0x31 contents: $ORIG31

/sbin/setVGAreg -u CRTC 0x31 0x00

/usr/bin/X11/XF86_SVGA

/sbin/setVGAreg -u CRTC 0x31 $ORIG31

As an example of the `idx' and `io' pseudo-registers, here's two equivalent

commands:

> setVGAreg CRTC 0x09 0xeb

is equivalent to

> setVGAreg idx 0x3D4 0x09 0xeb

and

> getVGAreg MISC

is equivalent to

> getVGAreg io 0x3CC

WARNING: since different SVGA cards use different extra address ranges in any

of the register sets, no checking is done to make sure you don't attempt to change

a non-existing register!

This is just a hacking tool! Use at your own risk. It was NOT intended to be

idiot proof! If you don't understand all this, then don't bother trying to use it.